# Microbiology of the food chain: Horizontal method for the detection, enumeration and serotyping of Salmonella – Detection of Salmonella spp. AS 5013.10-2022

This standard is an adoption with national modification of ISO 6579-1:2017. Microbiology of the food chain – Horizontal method for the detection, enumeration and serotyping of Salmonella – Part 1: Detection of Salmonella spp. and its Amendment No. 1. This standard replaces AS 5013.10.2009.

## SCOPE

This method is applicable to:

- products intended for human consumption (including raw meats and carcass swab and rinse samples) and the feeding of animals.

- environmental samples in the area of food production and food handling.

## PRINCIPLES

Salmonella Hofit is used as the positive control for this method. The detection of Salmonella spp. is broken down into four stages:

### Pre-enrichment in non-selective liquid medium

For meat and meat products a 1:10 dilution of the sample is enriched in buffered peptone water at 36 ± 2C for 18 h ± 2 h. Buffered peptone water should be warmed to room temperature or to 36 ± 2C for large volumes (i.e. >225 mL). For carcass sponges, buffered peptone water is added to the moistened sponge to bring the total volume to 60-100 ml and the sample incubated at 36 ± 2C for 18 h ± 2 h. In the case of sponges BPW need not be warmed to room temperature before being used to re-hydrate the sponge, for all subsequent additions BPW should be warmed to room temperature.

### Enrichment in selective liquid medium

Culture from the pre-enrichment broth is inoculated into Rappaport-Vassiliadis medium with soya (RVS broth) and Muller-Kauffmann tetrathionate/novobiocin broth (MKTTn broth, pH 8.0 ± 0.2 at 25C). The RVS broth is incubated at 41.5 ± 1C for 24 h ± 3 h and the MKTTn broth at 37 ± 1C for 24 h ± 3 h.

### Plating out and identification

Cultures obtained from the selective enrichment are streaked onto two selective media:

- Xylose lysine deoxycholate agar (XLD agar)

- And, for testing as part of export certification, any other solid selective medium that is complementary to XLD and able to detect H2S negative serovars of Salmonella e.g. Brilliant Green Agar (BGA).

XLD agar is incubated at 37 ± 1C and examined after 24 h ± 3 h. The second agar is incubated according to the manufacturer’s recommendations. The department does not require the use of duplicate 90 to 100 mm Petri dishes or a single 140 mm Petri dishes. Single 90 to 100 mm Petri dishes can be used. Confirmation can be directly off the selective agar if well isolated colonies are available.

### Confirmation of Salmonella

Colonies (maximum of 20) of presumptive Salmonella (subcultured on to nutrient agar if necessary) are confirmed by appropriate biochemical tests, as detailed in AS 5013.10 (2022). Preliminary confirmation at the isolating laboratory should include polyvalent O and H antisera. Rapid biochemical identification kits described in AOAC 978.24, AOAC 989.12, AOAC 991.13 and AOAC 2017.09 can be used for biochemical confirmation (section 9.5.3 of AS 5013.10). Salmonella isolates must be sent to a reference laboratory for serotyping.

## CHECKLIST

| Pre-enrichment | Is the buffered peptone water warmed to room temperature (to 36 ± 2C for large quantities)? |  |
| --- | --- | --- |
|  | Is the correct amount of enrichment broth used for the weight of sample analysed? |  |
|  | Is primary enrichment at 37 ± 1C for 16-20h? |  |
|  | Is a positive control run with each batch of samples analysed? |  |
|  | Are reference cultures inoculated into primary enrichment broth at a level of 10 to 100 cells? |  |
| Selective-enrichment | Is RVS broth sterilised at 115 C for 15 minutes? |  |
|  | Is MKTTn broth boiled not autoclaved? |  |
|  | Is RVS incubated at 41.5 ± 1C for 24 ± 3 h? |  |
|  | Is MKTTN incubated at 37 ± 1C for 24 ± 3 h? |  |
|  | Are all complete selective liquid media prepared on the day of use (or is a validated shelf-life provided by the manufacturer)? |  |
| Selective plating | What agars are used for isolation of suspect colonies? |  |
|  | Is the isolation of H2S negative strains considered in the laboratories methods manual and procedures? |  |
| Confirmation | How are cultures obtained for biochemical tests (if not streaked onto Nutrient agar is a purity check carried out)? |  |
|  | Are approved rapid bio-chemical test kits used? |  |
|  | Does preliminary confirmation at the isolating laboratory include polyvalent O and H antisera? |  |
|  | Are biochemical tests used sufficient to identify Salmonella spp.? |  |
|  | Are all suspect Salmonella sent to a reference Laboratory to be serotyped? |  |